# Deformable Sensors based on Architectured 2D Materials

We have demonstrated a stretchable photosensor based on crumpled graphene which exhibits enhanced and strain-tunable photoresponsivity (Advanced Materials 28, 4639 (2016); Nanoscale 9, 4058 (2017); Advanced Functional Materials 33, 1902216 (2019)). Crumple density, height, and pitch are modulated using applied strain and the crumpling is fully reversible during cyclical stretching and release, introducing a new capability of strain-tunable photoabsorption enhancement (Figure 5). Our work demonstrates a robust approach for stretchable and deformable graphene photodetector devices and showcases advanced functionalities where photosensitivity is modulated by mechanical deformation. We are also exploring controlled incision/cutting (i.e., kirigami) inspired wearable sweat sensors (Figure 5) based on graphene devices (Materials Today 34, 58 (2020)).